ECE-111 Advanced Digital Design Projects

Homework-4:

- Develop SystemVerilog RTL model for 4-bit Barrel Shifter using 2x1 Mux behavioral model

- Same Barrel Shifter RTL model should support left shift, right shift, left rotate and right rotate operation

- Use 2x1 Mux behavioral RTL model (will be provided) to design barrel shifter (Do not use 4x1 Mux model)

- Synthesize Barrel Shifter and review synthesis results (resource usage and RTL netlist/schematic)

- Run simulation using testbench provided and review waveform to confirm :

- left shift, right shift, left rotate and right rotate operation of barrel shifter RTL model behavior

- Assume below mentioned primary port names and SystemVerilog RTL module name as barrel shifter

Barrel Shifter

shift_value[1:0]

select

dout[3:0]

din[3:0]

direction

- Primary Ports for Barrel Shifter

- select : to select between shift or rotate operation

- select == 0 for shift operation

- select == 1 for rotate operation

- din : 4 bit input data

- dout : 4-bit output data

- direction : move bits in either left or right

- direction == 0, move bit to right

- direction == 1, move bit to left

- shift_value : bit positions to be shifted

- shift_value == 00, no shift operations

- shift_value == 01, move bits by 1-bit position

- shift_value == 10, move bits by 2-bit positions

- shift_value == 11, move bits by 3-bit positions

Homework Submission :

Submit report (PDF file) which should include:

- SystemVerilog design and testbench code

- Synthesis resource usage and schematic generated from RTL netlist viewer

- Simulation snapshot and explain simulation result to confirm it works as a barrel shifter with each type of operations (one example of each left shift, right shift, rotate left and rotate right)

- Resource usage explanation and post mapping schematic is optional to submit.

Note :

Remember bit inversion here does not mean negation of input values, it means swapping bit positions !

Example : if din = 0011  then after bit data inversion din should be = 1100